Telescope Use Regulations

Only the person authorized to use our telescope is permitted to use and control it.

“Warning”  - you never want to leave the scope tracking an object if you are leaving the scope for an extended time.    It will continue to try to follow the object below the horizon and the scope will hit the base, continue to run and strip the gears or burn out the motor when it is jammed up against the base.

To start your session of viewing you will want to “wake” the scope:

-You press “Undo”.

-Then you can select an object - like the moon, Jupiter, etc.  Because the scope has a GPS built into it, it knows that the moon or Jupiter will be in a different location than perhaps the night before when you last used the scope.    By selecting that object, the scope will slew to it. (You can use our directory for viewing objects or celestial areas or the directory from sky charts.)

-After viewing your first selection for viewing, you can enter other objects for the scope to “slew” to.

To finish your session:

-Press “Undo” several times until you are at the top of the menu.

-Then press the “Menu” button.

-Then the “up and down” buttons until you come to “Utilities” Press enter.

-Now press “Up and Down” until you come to “Hibernate”

(Follow instructions on hand-controller. )

-You’ll need to press “Enter” to have the scope hibernate.

It will stay in this position without slewing.

When you want to “wake” the scope, you’ll press “Undo”.  Then you can select an object again - like the moon, Jupiter, etc.  Because the scope has a GPS built into it, it knows that the moon or Jupiter will be in a different location than perhaps the night before when you last used the scope.    By selecting that object, the scope will slew to it.    This will be how you end a session remotely with the telescope and begin again on a following night.